Module 6 Notes

6.1 Wireless Networks

Network -  2+ devices connected through a communication medium. Powered by hardware and software.

Wireless Network – 2+ devices connected using radio waves powered by hardware and software.

Wi-fi – IEEE 802.11 network protocol family LANs (Local Networks)

Bluetooth – Short range (~30ft) PANs (personal Area Networks) Phones/laptops and their peripherals

RFID – Radio Frequency Identification – Passive vs active affects range. Toll booths, Inventory, Animal tracking chips

NFC – Near Field Communication – 1.5 inches or less. Credit cards, phones event tickets amiibo

IR – Infrared Light – Range is dependant. Remotes

Wireless network Types

WPAN (PAN) – Wireless Personal Area Network – Short range (typically Bluetooth) ~30ft Range, Room workspace/desk, small home

WLAN – Wireless Local Area Network – Mix of wired and wireless connection points. Range can be a single room to a large building or even a campus (CAN)

Wireless Access points (WAPs) – Connect a wireless user to a wired network

WWLAN – Wireless Wide Area Network – Connects WLANs/LANs/CANs across a large geographic area using radio waves

Range?

Radio Spectrum – lower frequency = Longer range but lower data rates

Receiver Sensitivity – The minimum signal strength that a receiver can interpret

`	Transmit Power – Higher transmit power gives larger range but increases power consumption

Antenna Gain – Converting radio waves into electrical Power. Affects transmission and reception by its location size design and directional or omnidirectional

Path Loss – Reduction in signal strength as the radio wave propagates Affected by distance environment and obstacles.

Benefits of Wireless

Convenience

Mobility

Productivity

Easy to set up

Expansion easier

Costs less

Disadvantages

Security Easy to intercept

Reliability

Interference

Speed is lower

Complexity in managing performance

6.2 Attacking Wireless Networks

Wireless Networks are easy to target as isolation of the network is difficult it is easy to access

Not broadcasting an SSID is security through obscurity don’t purely rely on it

Man-in-the-Middle Attacks – Easy to intercept communication

Impersonation/spoofing – tricking someone to connect to a rogue WAP “Pineapple router” or Evil Twin

Disassociation Attacks – Maliciously cloning a MAC address and sending a Disassociation frame to the WAP either for DoS purposes or to redirect to a malicious WAP or to collect Authentication traffic

Jamming attacks – DoS attack causing interference with the wireless signal

Encryption cracking – done on weak protocols like WEP and weak keys

6.3 Securing Wireless Networks

Encryption

WEP – Wired Equivalent Privacy – Small Key Space Vulnerable!

WPA- Wi-Fi Protected Access – Uses TKIP(Temporal Key Integrity Protocol) to strengthen WEP Also vulnerable now

WPA2 – Wi-Fi Protected Access v2 – uses advanced Encryption Standard (AES) safe

WPA3 – Wi-fi Protected Access v3 – uses Wi-Fi Divice Provisioning Protocol (DPP)  No need to transmit passwords uses QR codes or NFC tags Stronger Encryption (256-bit Galois/Counter mode Protocol (GCMP-256)

WPA2/3 Authentication

PSK – Personal - Pre-shared Key – Use of shared password

EAP – Enterprise- Extensible Authentication Protocol – Authentication database Username and unique password

Prevention of attacks

Power level configuration

WAP power/signal Adjustments will affect range Range affects access

Location of WAP– Placement adjustments affects range. Range affects access. Block signals with special paints from leaving

MAC Filtering- Only allow authorized devices Not foolproof MAC spoofing is possible and easy obstacle not a prevention

Static IPs required -  Required  static IP instead of dynamic Obsticle for the attacker

Disable SSID Broadcasts – Security Through obscurity. Can be good or bad thing

Limit device Capability – Turn off services you don’t need.

Trust- if you don’t trust the network don’t connect to it

VPN – Extra layer of protection especially on questionable pr public wifi networks